Table 1 Inhalation therapy devices used in out-patient care

| Device type | How do they work |
| --- | --- |
| Nebulisers, SVNs | Solution of a drug is nebulised in fine vapour with the mean of ultrasonics or air jet |
| MDIs* | Solution or suspension of a drug is formulated in propellant mix, under pressure, with the medicine being expelled (by a valve) in a metered volume from the volatile mixture as the propellant evaporates |
| DPIs | Compacted powder of a medicine is broken with the force of inhalation in the process called de-agglomeration into fine particles that are inhaled |
| Vaporisers | Formulations like oil, wax or plant material are heated without combustion. Active agents are released into a vapour that is inhaled by patient. |

DPIs – dry powder inhalers; MDIs- metered-dose inhalers; SVNs – small volume nebulisers

*MDIs are also referred as pMDIs (“pressurised metered-dose inhalers”)